TECHNICAL SHEET: Alfa Romeo 156 Super 2000

| Engine | Front transversal, 4 cylindres in line |
| --- | --- |
| Capacity | 1.998 cc |
| Power | 275 cv at 8.450 rpm |
| Fuel feed | Electronic system integrated with injection |
| Intake | Single throttle diam. 64 mm |
| Traction | Front wheel drive |
| Clutch | Carbon double discs diam. 184 mm |
| Gearbox | 6 gear + rev, sequential, front engagement with cut-off |
| Differential | N.Technology mechanic limited slip differential |
| Brakes | Front: 330 mm ventilated discs, 4 pots calipers Rear: 278 mm discs, 2 pots calipers Brake bias adjustable on pedal box Rear brake bias adjustable by driver |
| Steering | Rack and pinion, with electric pump servo assistance |
| Suspensions | 3 ways adjustable shocks absorbers Front: Double “A” arms Rear: McPherson strut |
| Wheels | Aluminium alloy wheels 9 x 17 |
| Aerodynamics | Front splitter (80mm from the ground) Tunable rear spoiler |
| Weight | 1.140 kg (driver included) |

TECHNICAL SHEET: Alfa Romeo 156 GTA Super 2000

| Engine | Front transversal, 4 cylindres in line |
| --- | --- |
| Capacity | 1.998 cc |
| Power | 260 cv at 8.450 rpm |
| Fuel feed | Electronic system integrated with injection |
| Intake | Single throttle diam. 64 mm |
| Traction | Front wheel drive |
| Clutch | Single disc diam. 200 mm |
| Gearbox | 5 gear + rev, front engagement |
| Differential | N.Technology mechanic limited slip differential |
| Brakes | Front: 295 mm discs, 4 pots calipers Rear: 278 mm discs, 2 pots calipers Brake bias adjustable on pedal box Rear brake bias adjustable by driver |
| Steering | Rack and pinion, with electric pump servo assistance |
| Suspensions | 4 ways adjustable shocks absorbers Front: Double “A” arms Rear: McPherson strut |
| Wheels | Aluminium alloy wheels 9 x 17 |
| Aerodynamics | Front splitter (80mm from the ground) Tunable rear spoiler |
| Weigth | 1.140 kg (driver included) |

TECHNICAL SHEET: Alfa Romeo 156 D2

| Engine | Front transversal, 4 cylindres in line, 16 V |
| --- | --- |
| Capacity | 1.997 cc |
| Power | 310 cv at 8.200 rpm |
| Fuel feed | Electronic system integrated with injection |
| Intake | 4 throttles |
| Traction | Front wheel drive |
| Clutch | Carbon triple discs |
| Gearbox | 6 gear + rev, sequential, front engagement with cut-off |
| Differential | Viscous / multiplates limited slip differential |
| Brakes | Front: 380 mm ventilated discs, 8 pots calipers Rear: 278 mm ventilated discs, 2 pots calipers Brake bias adjustable on pedal box Rear brake bias adjustable by driver |
| Steering | Rack and pinion, with electric pump servo assistance |
| Suspensions | 4 ways adjustable shocks absorbers Front: Double “A” arms Rear: McPherson strut |
| Wheels | Aluminium alloy wheels 9 x 19 |
| Aerodynamics | Adjustable front splitter Adjustable rear spoiler |
| Weigth | 975 kg |

TECHNICAL SHEET: Alfa Romeo 147 Superproduzione

| Engine | Front transversal, 4 cylinders in line 16V twin spark |
| --- | --- |
| Capacity | 1.970 cc |
| Power | 220 cv at 7.500 rpm |
| Fuel feed | Electronic system integrated with injection |
| Traction | Front wheel drive |
| Clutch | Single plate sintered |
| Gearbox | N.Technology 5 speed + rev, front engagement, H pattern |
| Differential | N.Technology mechanic limited slip differential |
| Brakes | Front: 295 mm discs Rear: 278 mm discs |
| Steering | Rack and pinion, with electric pump servo assistance |
| Suspensions | Front: double “A” arms Rear: McPherson strut |
| Wheels | Aluminium alloy wheels 7 x 15 |
| Weigth | 1.110 kg |

TECHNICAL SHEET: Alfa Romeo 147 GTA Cup

| Engine | Front transversal, 4 cylinders in line 16 v. twin spark |
| --- | --- |
| Capacity | 1.970 cc |
| Power | More than 220 cv |
| Fuel feed | Electronic system integrated with injection |
| Intake | Dynamic air duct |
| Traction | Front wheel drive |
| Clutch | Single plate |
| Gearbox | N.Technology 6 speed + rev, sequential, front engagement with cut-off |
| Differential | N.Technology mechanic limited slip differential |
| Brakes | Front: 305 mm discs, 4 pots calipers Rear: 251 mm discs, 1 pot callipers |
| Steering | Rack and pinion, with hydraulic servo assistance |
| Suspensions | Adjustable shock absorbers Front: double “A” arms, anti roll bar diam. 28 mm Rear: McPherson struts, adjustable anti roll bar diam. 18 mm |
| Wheels | Aluminium alloy wheels 9 x 17 dry 8 x 17 wet |
| Special parts | Digital integrated dashboard, lightened chassis, front adjustable spoiler, rear spoiler, lexan glasses, termic windscreen, N.Technology integrated roll bar |
| Weigth | 1.000 kg |

TECHNICAL SHEET: Alfa Romeo 147 Cup

| Engine: | Front transversal, 4 cylinders in line 16 v. twin spark |
| --- | --- |
| Capacity: | 1.970 cc |
| Power: | 200 cv at 7.250 rpm |
| Fuel feed: | Electronic system integrated with injection |
| Intake: | Dynamic air duct |
| Traction: | Front wheel drive |
| Clutch: | Single plate |
| Gearbox: | N.Technology 6 speed + rev, sequential, front engagement with cut-off |
| Differential: | N.Technology mechanic limited slip differential |
| Brakes: | Front: 278 mm discs, 4 pots calipers Rear: 251 mm discs, 1 pot callipers |
| Steering: | Rack and pinion, with hydraulic servo assistance |
| Suspensions: | Front: double “A” arms, anti roll bar diam. 28 mm Rear: McPherson struts, anti roll bar diam. 18 mm |
| Wheels: | Aluminium alloy wheels 8 x 17 |
| Special parts: | Digital integrated dashboard, lightened chassis, front adjustable spoiler, rear spoiler, lexan glasses, termic windscreen, N.Technology integrated roll bar |
| Weight: | 1.000 kg |

TECHNICAL SHEET: Alfa Romeo GT M-Jet

| Engine | Front transversal, 4 cylinders in line, 16V, diesel |
| --- | --- |
| Capacity | 1.910 cc |
| Power | 230 at 4.250 rpm |
| Fuel feed | Multijet |
| Traction | Front wheel drive |
| Clutch | Cerametallic |
| Gearbox | 6 speed + rev, sequential |
| Differential | Mechanic limited slip differential |
| Brakes | Front: discs, 4 pots calipers Rear : discs, 1 pot caliper |
| Steering | Rack and pinion, with hydraulic servo assistance |
| Suspensions | Front: double “A” arms, anti roll bar Rear: McPherson struts, anti roll bar |
| Wheels | Aluminium alloy wheels 9 x 17 |
| Special parts | Digital integrated dashboard, lightened chassis, front adjustable spoiler, rear spoiler, FIA homologated integrated roll bar |
| Weigth | 1.060 kg |

TECHNICAL SHEET: Alfa Romeo 156 GTA M

| Engine | Front transversal , 6 cylinders V 60° |
| --- | --- |
| Capacity | 3.458 cc |
| Power | 300 CV at 6.800 rpm |
| Fuel feed | Integrated ignition injection system |
| Traction | Front wheel drive |
| Intake | Single throttle diam. 74 mm |
| Gearbox | 6 speed |
| Differential | N.Technology hydraulic limited slip differential |
| Brakes | Front: 332 mm discs, 4 pots increased calipers Rear: 250 mm discs, 1 pot calipers |
| Steering | Rack and pinion, with electrical servo assistance |
| Suspensions | Front: double “A” arms Rear: McPherson struts Racing set up: adjustable height, springs with helper |
| Wheels | Forged aluminium alloy wheels 8 x 19 Tyres 235/35ZR19 |
| Aerodynamics | Front spoiler Rear spoiler |
| Weigth | 1.380 Kg |

TECHNICAL SHEET: Alfa Romeo 147 Cup

| Engine | Front transversal, 4 cylindres in line, 16V, twin spark |
| --- | --- |
| Capacity | 1.970 cc |
| Power | 210 cv at 7.500 rpm |
| Fuel feed | Electronic system integrated with injection |
| Traction | Front wheel drive |
| Clutch | Single disc |
| Gearbox | N.Technology 6 gear + rev, sequential, front engagement |
| Differential | N.Technology mechanic limited slip differential |
| Brakes | Front: 355 mm discs, 4 pots calipers Rear: 278 mm discs, 2 pots calipers |
| Steering | Rack and pinion, with electric pump servo assistance |
| Suspensions | Front: double “A” arms, 30 mm diameter anti roll bar with adjustable levers Rear: McPherson strut, 18 mm diameter adjustable anti roll bar |
| Wheels | Aluminium alloy wheels 8 x 18 |
| Aerodynamics | Digital integrated dashboard, lightened chassis, heated windscreen, N.Technology integrated roll bar |
| Weigth | 1.075 kg |