|  | # Connector A [8 pins] |
| --- | --- |
|  | A1 CAN BUS (B) |
|  | A2 BOSE Amplifier Input |
|  | A3 CAN BUS (A) |
|  | A4 N/C (w/manual says lights) - Entry of the contact {+15} ??? |
|  | A5 12V Switched for Aerial |
|  | A6 NC |
|  | A7 +12v Constant |
|  | A8 Ground [Earth] |
|  | Connector B Speakers [8 Pins] |
|  | B1 - Loudspeaker Rear Right {+} |
|  | B2 - Loudspeaker Rear Right {-} |
|  | B3 - Loudspeaker Front Right {+} |
|  | B4 - Loudspeaker Front Right {-} |
|  | B5 - Loudspeaker Front Left {+} |
|  | B6 - Loudspeaker Front Left {-} |
|  | B7 - Loudspeaker Rear Left {+} |
|  | B8 - Loudspeaker Rear Left {-} |
|  | Connector C [20 pins] |
|  | C1 - 5 Not Connected |
|  | C6 - 12V Switchable for BOSE amplifier |
|  | C7-9 Not connected |
|  | C10 Remote Control Signal A |
|  | C11 Remote Control Signal B |
|  | C12 Signal Earth |
|  | C13 ASCI-BUS in (for CD Changer) |
|  | C14 ASCI-BUS out (for CD Changer) |
|  | C15 12V Permanent (for CD Changer) |
|  | C16 12v Switchable (for CD Changer) |
|  | C17 ASCI-BUS Earth |
|  | C18 NF-Input Earth (for CD Changer) |
|  | C19 Input L (for CD Changer) |
|  | C20 Input R (for CD Changer) |
|  | Connector D [18 pins] |
|  | D1 - No Connection |
|  | D2 - No Connection |
|  | D3 - No Connection |
|  | D4 - No Connection |
|  | D5 - No Connection |
|  | D6 - Hook - Receiver Hang-Up Trigger [?mute?] |
|  | D7 - Audio Signal {Out Towards Built-In Telephone Receiver} |
|  | D8 - Microphone Input {From Built-In Telephone Receiver} |
|  | D9 - Microphone Input |
|  | D10 - No Connection |
|  | D11 - No Connection |
|  | D12 - Gnd [Earth] |
|  | D13 - No Connection |
|  | D14 - No Connection |
|  | D15 - No Connection |
|  | D16 - Audio Indicator - Audio Input From Built-In phone Receiver |
|  | D17 - Microphone Indicator - To Built-In Telephone Receiver |
|  | D18 - Microphone Indicator |