# Version with Heat - flange with VGT

Pin Out

- A1 - Chassis earth.

- A2 - Chassis earth.

- A3 - Chassis earth.

- A4 - Fuel pump relay coil supply.

- A5 - Power supply from main relay.

- A6 - Not connected.

- A7 - Not connected.

- A8 - Not connected.

- A9 - C-Can L.

- A10 - C-Can H.

- A11 - Fiat CODE control unit.

- A12 - Not connected.

- A15 - Power supply to main relay coil.

- A14 - Preheating indicator control relay coil power supply.

- A15 - Heat flange relay coil power supply.

- A16 - VGT solenoid control.

- A17 - Not connected.

- A18 - Power supply to compressor relay coil.

- A19 - Power supply for engine cooling fan control relays.

- A20 - Power supply to engine cooling fan relay (low speed)

- A21 - Instrument panel (MIL).

- A22 - Not connected.

- A23 - Cruise Control activation warning light.

- A24 - Fuel pump relay coil supply.

- A25 - Not connected.

- A26 - Not connected.

- A27 - Not connected.

- A28 - Not connected.

- A29 - Not connected.

- A30 - Not connected.

- A31 - Not connected.

- A32 - Not connected.

- A33 - Not connected.

- A34 - Not connected.

- A35 - Not connected.

- A36 - Not connected.

- A37 - Not connected.

- A38 - Not connected.

- A39 - Not connected.

- A40 - Not connected.

- A41 - Not connected.

- A42 - Not connected.

- A43 - Not connected.

- A44 - Not connected.

- A45 - Not connected.

- A46 - Not connected.

- A47 - Instrument panel (engine rpm).

- A48 - Tester input.

- A49 - From pulse generator (for speedometer).

- A50 - Not connected.

- A51 - 4 stage pressure switch.

- A52 - 4 stage pressure switch.

- A53 - Cruise Control ON.

- A54 - Vehicle brake light control.

- A55 - Cruise Control resume.

- A56 - Cruise Control set - .

- A57 - Cruise Control set +.

- A58 - From Fiat CODE control unit.

- A59 - Vehicle brake light control.

- A60 - 4 stage pressure switch.

- A61 - Sensor on clutch pedal.

- A62 - Not connected.

- A63 - Not connected.

- A64 - Not connected.

- A65 - Not connected.

- A66 - Not connected.

- A67 - Not connected.

- A68 - Not connected.

- A69 - Not connected.

- A70 - Not connected.

- A71 - Not connected.

- A72 - Not connected.

- A73 - Not connected.

- A74 - Not connected.

- A75 - Not connected.

- A76 - Potentiometer on accelerator pedal.

- A77 - Potentiometer on accelerator pedal.

- A78 - Potentiometer on accelerator pedal.

- A79 - Potentiometer on accelerator pedal.

- A80 - Potentiometer on accelerator pedal.

- A81 - Potentiometer on accelerator pedal.

- B82 - Fuel temperature sensor.

- B83 - Fuel temperature sensor.

- B84 - Engine coolant temperature sensor.

- B85 - Engine coolant temperature sensor.

- B86 - Air pressure and temperature sensor.

- B87 - Not connected.

- B88 - Not connected.

- B89 - Not connected.

- B90 - Fuel pressure sensor.

- B91 - Fuel pressure sensor.

- B92 - Fuel pressure sensor.

- B93 - Air pressure and temperature sensor.

- B94 - Air pressure and temperature sensor.

- B95 - Air pressure and temperature sensor.

- B96 - Not connected.

- B97 - Not connected.

- B98 - Not connected.

- B99 - Engine rpm sensor.

- B100 - Engine rpm sensor.

- B101 - Engine rpm sensor.

- B102 - Timing sensor.

- B103 - Timing sensor.

- B104 - Timing sensor.

- B105 - Not connected.

- B106 - Not connected.

- B107 - Not connected.

- B108 - Fuel pressure regulator.

- B109 - Fuel pressure regulator.

- B110 - Fuel filter heater relay control.

- B111 - 3rd pump element cut-out solenoid.

- B112 - Not connected.

- B113 - Not connected.

- B114 - Cylinder no. 4 injector.

- B115 - Not connected.

- B116 - Not connected.

- B117 - Cylinder no. 1 and no. 2 injector return

- B118 - Cylinder no. 3 and no. 4 injector return

- B119 - Cylinder no. 1 injector.

- B120 - Cylinder no. 2 injector.

- B121 - Cylinder no. 3 injector.

Injection system wiring diagram with Heat - flange with VGT

- 1 - Fuel temperature sensor.

- 2 - Engine coolant temperature sensor.

- 3 - Air pressure and temperature sensor.

- 4 - Fuel pressure sensor.

- 5 - Engine rpm sensor.

- 6 - Timing sensor.

- 7 - Fuel pressure regulator.

- 8 - Fuel filter heater relay.

- 9 - 3rd pump element cut-out solenoid.

- 10 - Cylinder no. 1 injector.

- 11 - Cylinder no. 2 injector.

- 12 - Cylinder no. 3 injector.

- 13 - Cylinder no. 4 injector.

- 14 - 4 stage pressure switch.

- 15 - Engine cooling relay (high speed)

- 16 - 40 A fuse.

- 17 - Engine cooling fan.

- 18 - Engine cooling relay (high speed)

- 19 - Resistor.

- 20 - Potentiometer on accelerator pedal.

- 21 - Engine cooling relay (low speed)

- 22 - 40 A fuse.

- 23 - Compressor.

- 24 - Sensor on clutch pedal.

- 25 - Compressor control relay.

- 26 - Heat - flange control relay.

- 27 - 7.5 A fuse.

- 28 - Inertia switch.

- 29 - Electric pump submerged in tank.

- 30 - VGT solenoid valve.

- 31 - Pulse generator.

- 32 - Relay controlling electric pump submerged in tank.

- 33 - Vehicle brake light control.

- 34 - 15 A fuse.

- 35 - 7.5 A fuse.

- 36 - Preheating warning light.

- 37 - Injection system failure warning light.

- 38 - 7.5 A fuse.

- 39 - 15 A fuse.

- 40 - 10 A fuse.

- 41 - 20 A fuse.

- 42 - Main relay.

- 43 - 7.5 A fuse.

- 44 - 20 A fuse.

- 45 - Ignition switch.

- 46 - Tester input.

- 47 - Chassis earth.

- 48 - Fiat CODE control unit.

- 49 - Signal for rev counter on instrument panel.

- 50 - Cruise control

- 51 - 7.5 A fuse.

- 52 - Cruise Control activation warning light.

- 53 - 150 A fuse.

- 54 - C-Can L network.

- 55 - C-Can H network.

# Version with thermal starter with VGT

Pin Out

- A1 - Chassis earth.

- A2 - Chassis earth.

- A3 - Chassis earth.

- A4 - Fuel pump relay coil supply.

- A5 - Power supply from main relay.

- A6 - Not connected.

- A7 - Not connected.

- A8 - Not connected.

- A9 - C-Can L.

- A10 - C-Can H.

- A11 - Fiat CODE control unit.

- A12 - Not connected.

- A15 - Power supply to main relay coil.

- A14 - Preheating indicator control relay coil power supply.

- A15 - Power supply to thermal starter relay coil.

- A16 - VGT solenoid control.

- A15 - Power supply to thermal starter solenoid coil.

- A18 - Power supply to compressor relay coil.

- A19 - Power supply for engine cooling fan control relays.

- A20 - Power supply to engine cooling fan relay (low speed)

- A21 - Instrument panel (MIL).

- A22 - Not connected.

- A23 - Cruise Control activation warning light.

- A24 - Fuel pump relay coil supply.

- A25 - Not connected.

- A26 - Not connected.

- A27 - Not connected.

- A28 - Not connected.

- A29 - Not connected.

- A30 - Not connected.

- A31 - Not connected.

- A32 - Not connected.

- A33 - Not connected.

- A34 - Not connected.

- A35 - Not connected.

- A36 - Not connected.

- A37 - Not connected.

- A38 - Not connected.

- A39 - Not connected.

- A40 - Not connected.

- A41 - Not connected.

- A42 - Not connected.

- A43 - Not connected.

- A44 - Not connected.

- A45 - Not connected.

- A46 - Not connected.

- A47 - Instrument panel (engine rpm).

- A48 - Tester input.

- A49 - From pulse generator (for speedometer).

- A50 - Not connected.

- A51 - 4 stage pressure switch.

- A52 - 4 stage pressure switch.

- A53 - Cruise Control ON.

- A54 - Vehicle brake light control.

- A55 - Cruise Control resume.

- A56 - Cruise Control set - .

- A57 - Cruise Control set +.

- A58 - From Fiat CODE control unit.

- A59 - Vehicle brake light control.

- A60 - 4 stage pressure switch.

- A61 - Sensor on clutch pedal.

- A62 - Not connected.

- A63 - Not connected.

- A64 - Not connected.

- A65 - Not connected.

- A66 - Not connected.

- A67 - Not connected.

- A68 - Not connected.

- A69 - Not connected.

- A70 - Not connected.

- A71 - Not connected.

- A72 - Not connected.

- A73 - Not connected.

- A74 - Not connected.

- A75 - Not connected.

- A76 - Potentiometer on accelerator pedal.

- A77 - Potentiometer on accelerator pedal.

- A78 - Potentiometer on accelerator pedal.

- A79 - Potentiometer on accelerator pedal.

- A80 - Potentiometer on accelerator pedal.

- A81 - Potentiometer on accelerator pedal.

- B82 - Fuel temperature sensor.

- B83 - Fuel temperature sensor.

- B84 - Engine coolant temperature sensor.

- B85 - Engine coolant temperature sensor.

- B86 - Air pressure and temperature sensor.

- B87 - Not connected.

- B88 - Not connected.

- B89 - Not connected.

- B90 - Fuel pressure sensor.

- B91 - Fuel pressure sensor.

- B92 - Fuel pressure sensor.

- B93 - Air pressure and temperature sensor.

- B94 - Air pressure and temperature sensor.

- B95 - Air pressure and temperature sensor.

- B96 - Not connected.

- B97 - Not connected.

- B98 - Not connected.

- B99 - Engine rpm sensor.

- B100 - Engine rpm sensor.

- B101 - Engine rpm sensor.

- B102 - Timing sensor.

- B103 - Timing sensor.

- B104 - Timing sensor.

- B105 - Not connected.

- B106 - Not connected.

- B107 - Not connected.

- B108 - Fuel pressure regulator.

- B109 - Fuel pressure regulator.

- B110 - Fuel filter heater relay control.

- B111 - 3rd pump element cut-out solenoid.

- B112 - Not connected.

- B113 - Not connected.

- B114 - Cylinder no. 4 injector.

- B115 - Not connected.

- B116 - Not connected.

- B117 - Cylinder no. 1 and no. 2 injector return

- B118 - Cylinder no. 3 and no. 4 injector return

- B119 - Cylinder no. 1 injector.

- B120 - Cylinder no. 2 injector.

- B121 - Cylinder no. 3 injector.

Diagram showing injection system with thermal starter with EGR

- 1 - Fuel temperature sensor.

- 2 - Engine coolant temperature sensor.

- 3 - Air pressure - temperature sensor in intake manifold.

- 4 - Fuel pressure sensor.

- 5 - Engine rpm sensor.

- 6 - Timing sensor.

- 7 - Fuel pressure regulator.

- 8 - Fuel filter heater relay.

- 9 - 3rd pump element cut-out solenoid.

- 10 - Cylinder no. 1 injector.

- 11 - Cylinder no. 2 injector.

- 12 - Cylinder no. 3 injector.

- 13 - Cylinder no. 4 injector.

- 14 - 4 stage pressure switch.

- 15 - Engine cooling relay (high speed)

- 16 - 40 A fuse.

- 17 - Engine cooling fan.

- 18 - Engine cooling relay (high speed)

- 19 - Resistor.

- 20 - Potentiometer on accelerator pedal.

- 21 - Engine cooling relay (low speed)

- 22 - 40 A fuse.

- 23 - Compressor.

- 24 - Sensor on clutch pedal.

- 25 - Compressor control relay.

- 26 - Thermal starter control relay.

- 27 - 7.5 A fuse.

- 28 - Inertia switch.

- 29 - Electric pump submerged in tank.

- 30 - Cruise Control activation warning light.

- 31 - Pulse generator.

- 32 - Relay controlling electric pump submerged in tank.

- 33 - Vehicle brake light control.

- 34 - 15 A fuse.

- 35 - 7.5 A fuse.

- 36 - Preheating warning light.

- 37 - Injection system failure warning light.

- 38 - 7.5 A fuse.

- 39 - 15 A fuse.

- 40 - 10 A fuse.

- 41 - 20 A fuse.

- 42 - Main relay.

- 43 - 7.5 A fuse.

- 44 - 20 A fuse.

- 45 - Ignition switch.

- 46 - Tester input.

- 47 - Chassis earth.

- 48 - Fiat CODE control unit.

- 49 - Signal for rev counter on instrument panel.

- 50 - Cruise control

- 51 - 7.5 A fuse.

- 52 - 150 A fuse.

- 53 - C-Can L network.

- 54 - C-Can H network.

- 55 - Thermal starter solenoid.

- 56 - Thermal starter solenoid control relay.

- 57 - VGT solenoid valve.